Genie Radio aid

This is the transmitter  (Tx) which the teacher wears

This is the receiver (Rx) which attaches to the hearing aids with the Y lead

Battery indicator

ndicator

Frequency -must be the same as receiver

r

Integral microphone

The Transmitter

On/off switch

Mute button

The aerial must be securely pushed into the Ex Mic socket of the transmitter

To connect the receiver to the hearing aids: …….. may need help attaching the Y lead from the receiver to the hearing aids.

Each hearing aid has a ‘shoe’ attached to the end. This has 3 holes in the bottom. Look carefully and you will see that one of the holes is bigger. Look at the 3 pins on the Y lead and push them into the shoe, with the fattest pin in the biggest hole. You will need to push gently as it is a tight fitting.

Please watch this excellent video which shows you how to operate and use the Genie radio aid:

https://www.youtube.com/watch?v=YkSfeG7V0DA

Using a Radio Hearing Aid – Good practice guidelines

- Wear the transmitter with the microphone a hand span distance from your mouth

- Remember to mute the transmitter or switch off when you are not speaking to the child or when you leave the room

- Don’t wear loose jewellery etc. that hits against the microphone.

- If using a TV, Video, computer or Soundfield system etc.  connect it to the transmitter (you can request a lead from the Sensory Support Service)

- Charge the batteries regularly – remember to turn off before charging.

- Check the equipment every day.

Daily checks

Check:

- both the transmitter and the receiver are turned on! Check again after breaks or lunch.

- the transmitter and receiver are both on the same frequency and that they are both turned on. Little fingers sometimes toggle with the channel change buttons!!

- the  battery indicator – does it have enough charge?

- the batteries are fitted correctly and examine for splits, cracks or damaged

- the mute button on the transmitter is turned off

- the aerial is fully inserted into the socket of the transmitter (and inserted in the correct socket!)

- the direct input leads on the receiver are fully inserted.

- the three pinned leads are fully inserted into the shoe of both hearing aids.

Any problems –

- Are the shoes fitting properly into the hearing aid?

- Are the receiver and the transmitter on the same frequency?

- Are the batteries charged? Try replacing the rechargeable batteries with AA battery – if this works the rechargeable batteries need replacing. Please contact the audiology technicians for new batteries. DO NOT ATTEMPT TO RECHARGE WHILST THE NON-RECHARGEABLE BATTERIES ARE INSERTED!!

- Is the aerial fully inserted into the socket?

If the problem persists, please contact the audiology technicians:

Tel: 0121 464 5603      email: Sharon.B.Flaherty@birmingham.gov.uk